1. ------IND- 2008 0407 CZ- EN- ------ 20081027 --- --- PROJET

Draft

regulation

dated ………2008

laying down a list of selected items in the nuclear sector

The State Office for Nuclear Safety specifies pursuant to § 47 Paragraph 7 implementing § 2(j) of Act No. 18/1997 Coll. on peaceful utilisation of nuclear energy and ionising radiation (the Atomic Act) and on amendments to other legislation:

§

Subject

This Regulation sets out in its Annex a list of selected items in the nuclear power sector.

§

Annulment Provision

Regulation No. 179/2002 Coll., which lays down a list of selected items and dual-use items in the nuclear sector, is hereby annulled.

§

Effectiveness

The Regulation enters force on …………

Chair:

Ing. Drábová

‘Annex to Regulation No. …/2008 Coll.’

##### LIST OF SELECTED ITEMS

(SELECTED MATERIALS, EQUIPMENT AND TECHNOLOGY 
IN THE NUCLEAR SECTOR)

##### SUBJECT TO IMPORT, EXPORT AND TRANSIT CONTROLS

The list has been drawn up according to the

International Agency Atomic Energy document

INFCIRC / 254 / Rev. 9 / Part 1

CONTENTS

0.	Wrapped assemblies for irradiated nuclear fuel and hot chambers

1.	Nuclear reactors and specially designed or prepared equipment and components therefor

2.	Non-nuclear materials for reactors

3.	Plants for the reprocessing of irradiated fuel elements and equipment specially designed or prepared therefor

4. 	Plants for the fabrication of nuclear reactor fuel elements and equipment specially designed or prepared therefor

5.	Plants for the separation of isotopes of natural uranium, depleted uranium or special fissionable material and equipment other than analytical instruments, specially designed or prepared therefor

- Plants for the manufacture and reworking of concentrations of heavy water, deuterium and its compounds and devices specially designed or prepared therefor

7.	Plants for the conversion of uranium and plutonium for use in the manufacture of fuel elements and the separation of the uranium isotopes specified in Parts 4 and 5, and equipment specially designed or prepared therefor

8.	General notes, technical controls and definitions

0.	Wrapped assemblies for irradiated nuclear fuel and hot chambers

0.1  Wrapped assemblies for irradiated fuel

Wrapped assemblies for the transportation and/or storage of irradiated nuclear fuel, which provides chemical, thermal and radiation protection, and removal of decay heat during handling, transportation and storage.

0.2  Hot chambers

Hot chambers or interconnected hot chambers with a total volume of at least 6 m3 and shielding corresponding to the equivalent of 0.5 m or more concrete, with a density of 3.2 g/cm3 or higher, with facilities for remote control.

1.	Nuclear reactors and specially designed or prepared equipment and components therefor

1.1  Complete nuclear reactors

Nuclear reactors able to sustain a controlled fission chain reaction other than zero-power reactors. Zero-power reactors are defined as reactors designed with a maximum annual production of plutonium not exceeding 100 g.

#### Explanatory note

‘Nuclear reactor’ includes items that are located within a reactor vessel or that are directly connected to it, equipment that manages core output, and components that contain the coolant of the primary circuit of reactor, come into direct contact with it, or control its circulation. Reactors that can be modified so that they produce significantly more than 100 grams of plutonium annually cannot be excluded. Reactors designed for continuous operation at a significant level of performance (which require a system of active cooling of the core), regardless of their plutonium production capacity, are not considered to be ‘zero-power reactors’.

EXPORTS 

The export of the whole set of major items within this boundary will take place only in accordance with the procedures set out in Part 8. The individual items within a functionally-defined framework, which can be exported only in accordance with the procedures set out in Part 8, are listed in Paragraphs 1.2 to 1.10. The government reserves the right to apply the procedures set out in Part 8 to other items in the functionally-defined framework.

1.2  Reactor vessels

Metal vessels or major shop-fabricated parts specially designed or prepared for location in the core of a nuclear reactor as defined in Paragraph 1.1, as well as the nuclear reactor internals as defined in Paragraph 1.8.

Explanatory note 
The lid of the reactor vessel is included in Item 1.2 as a major shop-produced component of the reactor vessel.

1.3  Refuelling machines for nuclear reactors

Handling equipment specially designed or prepared for fuelling or removing fuel from a nuclear reactor as defined in Paragraph 1.1.

Explanatory note

The items listed above are capable of changing fuel during operation and using technically-complex elements to carry out location or direction that allows ongoing complex operations to be carried out when changing fuel during the shutdown of the nuclear reactor, where direct observation or access to the fuel is not usually possible.

1.4  Control rods of the nuclear reactor and related equipment

Specially constructed or modified rods, their load bearing or suspensive construction, traction bars and guide tubes for managing the fission process in a nuclear reactor as defined in Paragraph 1.1.

1.5  Pressure tubes of the nuclear reactor

Tubes that are specially designed or prepared to contain fuel elements and the primary coolant of the reactor as defined in Paragraph 1.1 and operating at a pressure of more than 50 atmospheres.

1.6  Zirconium tubes

Metallic zirconium and its alloys in the form of pipe or tube assemblies, specially designed or prepared for use in a nuclear reactor as defined in Paragraph 1.1, in quantities exceeding 500 kg for any one recipient country in any period of 12 months, for which the weight ratio of hafnium to zirconium less than 1:500.

1.7  Primary coolant pumps

Pumps specially designed or prepared for ensuring the circulation of the primary coolant of nuclear reactors as defined in Paragraph 1.1.

Explanatory note

Specially designed pumps may include elaborate sealing or multiple sealing systems designed to prevent leakage of primary coolant, hermetic motor pumps, and pumps with inertial mass systems. This definition includes pumps certified in accordance with Part III Section I Subsection NB (part of Class 1) of the American Society of Mechanical Engineers (ASME) Code, or equivalent standards.

1.8  Nuclear reactor internals

Nuclear reactor internals specially designed or prepared for use in a nuclear reactor as defined in Paragraph 1.1, including load support structures for the core, fuel channels, thermal shields, baffles, core grid plates and diffuser plates.

Explanatory note

‘Nuclear reactor internals’ are important structures within a reactor vessel, which have one or more functions such as supporting the core, maintaining fuel alignment, directing the flow of primary coolant, providing radiation shielding for the reactor vessel and guiding in-core instrumentation.

1.9  Heat exchangers

Heat exchangers (steam generators) specially designed or prepared for use 
in the primary coolant circuit of a nuclear reactor as defined in Paragraph 1.1.

Explanatory note

Steam generators are specially designed or prepared equipment for transferring heat generated in the reactor (primary side) to transform the feed water (secondary side) into steam. In a liquid metal fast breeder reactor in which an intermediate liquid metal coolant loop is also present, the heat exchangers for transferring heat between the primary side and the intermediate cooling circuit come within the framework of controlled items such as additional steam generator parts. The scope of control of this item does not include heat exchangers for emergency cooling systems or for decay heat cooling systems.

1.10 Equipment for the detection and measurement of neutrons

Specially designed or prepared equipment for the detection and measurement of neutrons to determine neutron flux levels inside a reactor core as defined in Paragraph 1.1.

#### Explanatory note

This item includes internal and external devices that measure neutron flux levels in a large range, usually from 104 neutrons per cm2/s to 1010 neutrons per cm2/s or more. Ex-core refers to the devices outside a reactor core as defined in Paragraph 1.1. that are located within the biological shielding.

2.	Non-nuclear materials for reactors

2.1  Deuterium and heavy water

Deuterium, heavy water (deuterium oxide) and other compounds of deuterium, in which the ratio of deuterium atoms to hydrogen atoms exceeds 1:5,000 for use in a nuclear reactor as defined in Paragraph 1.1, in quantities exceeding 200 kg of deuterium for any one recipient country in any period of 12 months.

2.2  Nuclear grade graphite

Graphite with a purity of more than 5 parts per million boron equivalent and density greater than 1.50 g/cm3 that is suitable for use in a nuclear reactor as defined in Paragraph 1.1, in quantities exceeding 30 t for any one recipient country in any period of 12 months.

Explanatory note

For the purpose of export control, the government determines whether or not the export of graphite corresponding to the above specification is intended for use in a nuclear reactor.

Explanatory note

The boron equivalent (BE) may be determined experimentally or calculated as the sum BEz for impurities (other than BEcarbon, since carbon is not considered to be an impurity) including boron, where:

BEz (ppm) = CF x concentration of element Z (in ppm);

CF is the conversion factor: (δZ x AB) divided by (δB x AZ);

δB and δZ are the effective thermal neutron capture cross-sections (in barns) for naturally-occurring boron and element Z respectively; and AB and AZ are the atomic masses of naturally-occurring boron and element Z respectively Z.

- Plants for the reprocessing of irradiated fuel elements and equipment specially designed or prepared therefor

Introductory note

By reprocessing irradiated nuclear fuel, plutonium and uranium are separated from the highly radioactive fission products and other transuranic elements. This separation can be achieved using various technical processes. Over the years, the Purex process has become the most widely used and recognised. Purex involves the dissolution of irradiated nuclear fuel in nitric acid, followed by separation of uranium, plutonium and fission products by solvent extraction, using tributyl phosphate in an organic solvent.

Purex plants use the following technical operations or similar: chopping irradiated fuel elements, dissolution of fuel, solvent extraction and process liquor storage. There might also be equipment for thermal denitration uranium nitrate, conversion of plutonium nitrate to oxide or metal, and for treatment of fission product waste liquor into a form that is suitable for long-term storage or disposal. But the specific type and configuration of the equipment carrying out these operations can vary between the various Purex plants, for these reasons: the type and quantity of irradiated fuel to be reprocessed and the intended destination of recovered materials, and the safety and maintenance philosophy incorporated into the plant design.

A ‘plant for the reprocessing of irradiated fuel elements’ includes equipment and components that normally come into direct contact with irradiated fuels, the major nuclear material and fission product processing streams, or that directly control them.

These processes, including the complete systems for the conversion of plutonium and plutonium metal production are closely linked with measures that prevent criticality (for example, by modifying geometric arrangements), exposure (for example, through shielding) and the risk of toxicity (such as the use of protective packaging).

EXPORTS 

The export of the whole set of major items within this boundary will take place only in accordance with the procedures set out in Part 8. The government reserves the right to apply the procedures set out in Part 8 to other items in the functionally-defined framework.

Items corresponding to the term ‘equipment specially designed or prepared’ for the reprocessing of irradiated fuel elements include:

3.1  Machines for chopping irradiated fuel elements

Introductory note

This device breaches the cladding of the fuel and thus prepares the irradiated nuclear material for dissolution. Specially designed mechanical shears are most commonly used, but modern equipment such as lasers may also be used.

Remotely operated equipment specially designed or prepared for use in the plant for reprocessing irradiated fuel elements, which are intended for cutting, chopping or shearing irradiated fuel assemblies, bundles or rods.

3.2  Solvent tank

Introductory note

Chopped-up spent fuel is normally sent to the solvent tank. In these vessels, secured against criticality, the irradiated nuclear material is dissolved in nitric acid and the remaining hulls are removed from the process stream.

Tanks secured against criticality (for example, a small diameter, annular, or slab design), specially designed or prepared for use in reprocessing plants, which are intended for dissolution of irradiated nuclear fuel and which are resistant to hot, highly corrosive liquid and can be remotely loaded and serviced.

3.3  Solvent extractors and solvent extraction equipment

Introductory note

Solvent extractors receive both the solution of irradiated fuel from the solvent tanks and organic solutions that separated uranium, plutonium and fission products. Equipment for solvent extraction is usually designed to meet strict operating parameters, such as a long operating life without maintenance requirements, easy replacement, simplicity of operation and control, and flexibility under changed technological conditions.

Specially designed or prepared extractors such as packed and pulse columns, mixing and settling tanks or centrifugal reactors, intended for use in plants for the reprocessing of irradiated fuel elements. Solvent extractors must be resistant to corrosion by nitric acid. Solvent extractors are normally produced under extremely high standards (including special welding, inspections, quality assurance and quality control) of low carbon stainless steel, titanium, zirconium and other high-quality materials.

  - Chemical holding or storage vessels

Introductory note

Extraction operations are based on three main process liquor streams. Chemical holding or storage vessels are used in the further processing of all three streams as follows:

a) Net uranium nitrate solution is concentrated by evaporation and passed to a denitration process where it is converted to uranium oxide. This oxide is re-used in the nuclear fuel cycle.

b) Highly radioactive fission products solution is normally concentrated by evaporation and stored as a liquid concentrate. This concentrate may be subsequently evaporated and converted to a form suitable for storage or disposal.

c) Pure plutonium nitrate solution is concentrated and stored until its transfer to the next step in the technological process. Specifically, the storage or holding vessel for plutonium solutions is designed to avoid criticality problems resulting from changes in concentration and form of the stream.

Specially designed or prepared holding or storage vessels intended for use in a plant for the reprocessing of irradiated fuel. The holding or storage vessels must be resistant to corrosion by nitric acid. They are usually made from materials such as low carbon stainless steel, titanium or zirconium, or other high-quality materials. Holding and storage vessels can be designed for remote operation and maintenance and may have the following parameters for the prevention of criticality:

(1) walls or internal structures correspond to a boron equivalent of at least 2 %; or

(2) a maximum diameter of 175 mm (7 in) for cylindrical vessels; or

(3) a maximum width of 75 mm (3 in) for each slab or annular vessel.

4.	Plants for the fabrication of nuclear reactor fuel elements and equipment specially designed or prepared therefor

Introductory note

Nuclear fuel elements are produced from one or more source or special fissionable materials. For oxide-based fuels, the most common type of fuel, this involves equipment for pressing pellets, sintering, grinding and grading. MOX fuels are handled in glove boxes (or similar areas) until they are hermetically sealed in cladding. In all cases, the fuel is hermetically sealed inside a suitable cladding, which is designed to be the primary envelope encasing the fuel so as to provide suitable hermetic sealing and removal of heat arising from the fuel in order to ensure the safe operation of the rector. Extremely high standards of precision control of processes, procedures and equipment are necessary in all cases in order to ensure the predictable and safe removal of heat from the fuel.

Items corresponding to the concept of ‘and equipment specially designed or prepared’ for the production of fuel elements include equipment that:

(a) normally comes into direct contact with, or directly processes, or controls, the production flow of nuclear material;

(b) hermetically seals the nuclear material inside the cladding;

(c) inspects the integrity of the cladding or hermetic seal; or

(d) inspects the finish treatment of the sealed fuel.

Explanatory note

Such equipment or systems of equipment may include: 
1) fully automated control stations specially designed or prepared for checking the final dimensions and surface defects of the fuel pellets; 
2) automatic welding machines specially designed or prepared for welding end covers of fuel elements (or rods); 
3) automatic test and control stations specially designed or prepared for monitoring the integrity of completed fuel elements (or pins).

Item 3 usually includes equipment for: 
(a) X-ray testing of element (or rod) and end cover welds; 
(b) helium leakage detection from pressurised elements (or rods); 
(c) gamma-scanning of the elements (or pins) in order to verify the correct loading of their fuel pellets.

5.	Plants for the separation of isotopes of natural uranium, depleted uranium or special fissionable material and equipment other than analytical instruments, specially designed or prepared therefor

Introductory note

Plants, equipment and technology for the separation of uranium isotopes in many cases are very closely linked with the plants, equipment and technology for the separation of stable isotopes. In specific cases, the controls under Part 5 are also used in the case of plants or equipment intended for the separation of stable isotopes. These inspections of plants or equipment intended for the separation of stable isotopes are complementary to inspections of plants or equipment specially designed or prepared for the processing, use or production of special fissionable material. These controls do not apply to the electromagnetic separation process.

Items that correspond to the term ‘equipment, other than analytical instruments, specially designed or prepared’ for the separation of isotopes of uranium include:

5.1  Gas centrifuges, assemblies and components specially designed or prepared for use in gas centrifuges

Introductory note

The gas centrifuge normally consists of a thin-walled cylinder(s) with a diameter of 75 mm (3 in) to 400 mm (16 in) placed in a vacuum environment and spun at high peripheral speed of the order of 300 m/s or more, around a vertical axis. In order to achieve such high speeds, construction materials of the rotating components of high strength relative to weight. The rotor assembly, and therefore its components, must be manufactured to very close tolerances in order to minimise imbalances during operation. In contrast with other centrifuges, the gas centrifuge for uranium enrichment is characterised by having within the rotor chamber a rotating disc baffle(s) and a stationary tube arrangement for feeding tube and extracting UF6 gas and featuring at least three separate channels, of which two are connected to scoops extending from the rotor axis towards the periphery of the rotor chamber. The vacuum also contains a number of critical items that do not rotate and that, although specially designed, are not difficult to produce and are not made of special materials. However, the gas centrifuge requires a large number of these components, so that their quantities can provide an important indication of end use.

5.1.1. Rotating components

(a) Complete rotor assemblies

Thin-walled cylinders, or a number of interconnected thin-walled cylinders, which are made from one or more of the materials with a high strength to density ratio, described in the explanatory note to this Section. If the cylinders are interconnected, they are connected by flexible bellows or rings as described in Part 5.1.1 (c). Rotor is equipped with an internal baffle(s) and end caps as described in Part 5.1.1 (d) and 5.1.1 (e). However, the complete assembly may be delivered only partly assembled.

(b) Rotor tubes

Specially designed or prepared thin-walled cylinders with a wall thickness of 12 mm (0.5 in) or less, with a diameter of 75 mm (3 in) to 400 mm (16 in) made from one or more of the materials with a high strength to density ratio as described in the explanatory footnote to this Section.

(c) Rings or bellows

Components specially designed or prepared to give localised support to the rotor tube or to join together a number of rotor tubes. The bellows is a coiled short cylinder with a diameter of 75 mm (3 in) to 400 mm (16 in) with a maximum wall thickness of 3 mm (0.12 in), made from one or more of the materials with high strength to density ratio as described in the explanatory note to this Section.

(d) Baffles

Disc-shaped components with a diameter of 75 mm (3 in) to 400 mm (16 in), specially designed or prepared to be fitted inside the centrifuge rotor tube, in order to isolate the take-off chamber from the main separation chamber and, in some cases, to assist the circulation of UF6 gas inside the main separation chamber of the rotor tube. They are made from one or more of the materials with high strength to density ratio as described in the explanatory note to this Section.

(e) Top/bottom end caps

Disc-shaped components with a diameter of 75 mm (3 in) to 400 mm (16 in) specially designed or prepared to fit the ends of the rotor tube and contain the UF6 within the rotor tube, and in some cases to support, retain or contain as an integral part an element of the upper bearing (top cap) or to carry the rotating elements of the motor and lower bearing (bottom cap). They are made from one of the materials with high strength to density ratio described in the explanatory note to this Section.

Explanatory note

The following materials are used in the rotating components of the centrifuge:

a) maraging steel whose ultimate tensile strength is 2.05 x 109 N/m2 (300,000 psi) or more;

b) aluminium alloys whose ultimate tensile strength is 0.46 x 109 N/m2 (67,000 psi) or more;

c) filametary materials suitable for use in composite structures with a specific modulus of 3.18 x 106 m or greater and a specific ultimate tensile strength of 7.62 x 104 m or greater (the ‘specific modulus’ is Young’s Modulus in N/m2 divided by the specific weight in N/m3; the ‘specific ultimate tensile strength is the ultimate tensile strength in N/m2 divided by the specific weight in N/m3).

5.1.2. Static components

(a) Magnetic suspension bearings

Specially designed or prepared bearing assemblies, consisting of annular magnets suspended within a housing containing a damping medium. The housing is made from UF6-resistant materials (see explanatory note to Section 5.2). Magnetic couples with a pole piece or a second magnet are attached to the top cap, as described in Section 5.1.1 (e). The magnet may be ring-shaped with a maximum ratio between the outer and inner diameter of 1.6:1. The magnet may have an initial permeability of 0.15 H/m (120,000 in CGS units) or more, a remanence of 98.5% or more, and an energy product greater than 80 kJ/m3 (107 gauss-oersteds). In addition to the usual material properties, it is a prerequisite that the magnetic axis deviation from the geometrical axes is limited to very small tolerances (less than 0.1 mm or 0.004 in) or that homogeneity of the material of the magnet is specially called for.

(b) Bearings/dampers

Specially designed or prepared bearings comprising a pivot/cup mounted on a damper. The pivot is usually a hardened steel shaft with a hemisphere at one end and a means of attachment to the bottom cap described in Section 5.1.1 (e) at the other. The shaft may also have a hydrodynamic bearing attached. The cup is pellet-shaped with a hemispherical indentation in one surface. These components are often supplied separately to the damper.

(c) Molecular pumps

Specially designed or prepared cylinders having internally machined or extruded helical grooves and internally-machined bores. Normal dimensions are as follows: inner diameter 75 mm (3 in) to 400 mm (16 in), wall thickness at least 10 mm (0.4 in), with the ratio of length to diameter of 1:1 or more. Grooves are typically rectangular in cross-section and 2 mm (0.08 in) or more in depth.

(d) Motor stators

Specially designed or modified ring-shaped stators for high-speed multiphase AC hysteresis (or reluctance) motors, prepared for synchronous operation within a vacuum in the frequency range of 600-2,000 Hz and a power range of 50-1,000 VA. The stators consist of multiphase windings on a low loss laminated core consisting of thin iron sheets, usually with a thickness of 2 mm (0.08 in) or less.

(e) Centrifuge housing

Components specially designed or prepared for the mounting of a gas centrifuge rotor tube assembly. The housing consists of a rigid cylinder with a wall thickness of up to 30 mm (1.2 in) with precision machined ends to locate the bearings and with one or more mounting flanges. The machined ends are parallel to each other and perpendicular to the longitudinal axis of the cylinder with a tolerance of 0.05º or less. The housings are made of materials that are resistant or protected by materials resistant to UF6 corrosion.

(f) Scoops

Tubes with an inside diameter of up to 12 mm (0.5 inch), specially designed or prepared for the extraction of UF6 gas from the rotor tube by a pitot tube action (with an aperture facing into the circumferential gas flow inside the rotor tube, for example, by bending the end of a radially placed tube) which can be fitted to the central gas extraction system. The pipes are made of or protected by materials resistant to corrosion by UF6..

5.2  Specially designed or introduced auxiliary systems, equipment and components for gas centrifuge enrichment plants

Introductory note

The auxiliary systems, equipment and components for a gas centrifuge enrichment plant are systems that secure the feeding of UF6 into the centrifuges, link individual centrifuges to each other to form cascades (or stages), allowing a gradual increase in enrichment and the extraction of 'product' and 'tails' UF6 from the centrifuges, along with the equipment needed to drive the centrifuges or to control the plant. UF6 is normally evaporated from the solid using heated autoclaves and is distributed in gaseous form to the centrifuges via cascade header pipework (collectors). The ‘product’ and ‘tails’ UF6 flowing from the centrifuges also pass through the cascade header pipework (collectors) to cold traps operating at about 203 K (-70°C), where they condense before being transferred into containers suitable for transportation or storage. Because an enrichment plant consists of many thousands of centrifuges arranged in cascades, there are many kilometres of cascade header pipework (collectors) containing thousands of welds with much repetition of layout. Equipment, components and piping systems are manufactured such that they meet the very high vacuum and cleanliness standards.

5.2.1. Feed systems/product and tails withdrawal systems

Specially designed or prepared process systems include:

(a) Feed autoclaves (or stations) used for passing UF6 to centrifuge cascades at up to 100 kPa (15 psi) and at a rate of 1 kg/h or higher.

(b) Desublimers (or cold traps) used to remove UF6 from the cascades at pressures of up to 3 kPa (0.5 psi) (desublimers can be cooled to a temperature of 203 K (-70°C) and heated to a temperature of 343 K (+70°C)).

(c) ‘Product’ and ‘tails’ used for trapping UF6 into containers.

The plant, equipment and pipework is wholly constructed of or lined with materials resistant to UF6 corrosion (see introductory note to this Section) and manufactured such that they meet the very high vacuum and cleanliness standards.

5.2.2  Machine header piping systems (collectors)

Specially designed or prepared piping systems and header systems (collectors) for handling UF6 within the centrifuge cascades. The piping network is normally of the ‘triple’ header system, where each centrifuge is connected to each of the headers (collectors). This arrangement is repeated many times. All these systems are constructed of materials resistant to corrosion by UF6 (see explanatory note to this Section) and manufactured such that they meet the very high vacuum and cleanliness standards.

5.2.3  Special shut-off and control valves

Specially designed or prepared bellows-sealed valves, manual or automated, shut-off or control, manufactured of or coated by materials resistant to corrosion by UF6 with a diameter of 10 to 160 mm for use in main or auxiliary systems of gas centrifuge enrichment plants.

5.2.4  UF6 mass spectrometers/ion sources

Specially designed or prepared magnetic or quadrupole mass spectrometers capable of taking ‘on-line’ samples of feed, product or tails from UF6 gas streams and having all of the following characteristics:

1. unit resolution for atomic mass unit greater than 320;

2. ion sources constructed of or lined with nichrome or monel or nickel plated;

3. electron bombardment ionisation sources;

4. collector system suitable for isotopic analysis.

5.2.5  Frequency changes

Frequency changers (also known as converters or inverters) specially designed or prepared to supply motor stators as defined under Paragraph 5.1.2 (d) or parts, components and sub-assemblies of such frequency changers having all of the following characteristics:

1. multiphase output in the frequency area 600-2,000 Hz;

2. high stability (with frequency control better than 0.1%);

3. low harmonic distortion (less than 2%); and

- efficiency exceeds 80%.

Explanatory note

The items listed above either come into direct contact with the UF6 process gas or directly control the centrifuges and the flow of gas from centrifuge to centrifuge and cascade to cascade. 

Materials resistant to UF6 corrosion include stainless steel, aluminium, aluminium alloys, nickel or nickel alloys containing a minimum of 60% nickel.

5.3  Specially designed or prepared assemblies and components for use in gaseous diffusion enrichment

Introductory note

In the gaseous diffusion method of uranium isotope separation, the main technological  assembly is a special porous gaseous diffusion barrier, heat exchangers for cooling the gas (which is heated by compression), seal values and control valves and, pipe networks. Whereas gas diffusion technology is based on the use of uranium hexafluoride (UF6), all equipment, pipeline and instrumentation surfaces (that come into contact with the gas) must be made of materials that remain stable when in contact with UF6. A gaseous diffusion plant requires a large number of these units, so that quantities may be an important indication of end use.

5.3.1. Gas diffusion barriers

(a) Specially designed or prepared thin, porous filters of pore size in the range of 100 to 1,000 Ǻ (angstroms), a thickness of 5 mm (0.2 in) or less, and for tubular forms, of a diameter of 25 mm (1 in) or less, made of metallic, polymer or ceramic materials resistant to corrosion by UF6.

(b) Specially prepared compounds or powders for the production of the filters. Such compounds and powders include nickel or its alloys with a minimum of 60% nickel, aluminium oxide, or UF6-fully resistant fluorinated hydrocarbon polymers with a purity of 99.9% or more, with a particle size of less than 10-5 m and a high degree of uniformity of particle size, which are specially adapted for the manufacture of gaseous diffusion barriers.

5.3.2  Diffuser housings

Specially designed or prepared hermetically-sealed cylindrical vessels with a diameter of more than 300 mm (12 in) and a height greater than 900 mm (35 in), or rectangular vessels of comparable dimensions, which have one inlet and two outlet connections, all of which have a diameter of more than 50 mm (2 in), for containing the gaseous diffusion barrier. These vessels are made of or lined with materials resistant to UF6 corrosion and are designed for horizontal or vertical installation.

5.3.3  Compressors and gas blowers

Specially designed or prepared axial, centrifugal or positive displacement compressors or gas blowers with a minimum suction volume capacity of 1 m3/min of UF6 and a discharge pressure of up to several hundred kPa (100 psi), designed for long-term operation in a UF6 environment with or without an appropriately-powered electrical motor, as well as separate assemblies of such compressors and gas blowers. These compressors and blowers have a pressure ratio of 2:1 to 6:1 and are made of or lined with materials resistant to UF6 corrosion.

5.3.4  Shaft seals

Specially designed or prepared vacuum seals, which ensure the sealing of input and output flanges, used for sealing the shaft connecting the compressor rotor or blowers with the drive motor and providing a reliable seal for the inner chamber compressor or blower, which is filled with UF6. Such seals are normally designed with a buffer gas in-leakage rate of less than 1,000 cm3/min (60 in3/min).

5.3.5  Heat exchangers for cooling UF6

Specially designed or prepared heat exchangers made of or lined with materials resistant to corrosion by UF6 (except stainless steel) or with copper and any combination of these metals, and intended for a leakage pressure change rate of less than 10 Pa (0.0015 psi) per hour under a pressure difference of 100 kPa (15 psi).

5.4  Specially designed or prepared auxiliary systems, equipment and components for use in gaseous diffusion enrichment plants

Introductory note

Auxiliary systems, equipment and components for enrichment plants using gaseous diffusion systems are the systems necessary to feed UF6 into separation units and the connection of the individual units between themselves create cascades (or stages), thus allowing for progressively higher enrichments and the extraction of the ‘product’ and ‘tails’ UF6 from the diffusion cascades. Due to the large inertial properties in the process of diffusion cascades, any interruption of their activities and especially their shut-down leads to serious consequences. Therefore, a strict and continuous maintenance of vacuum in all technological systems, automatic accident protection and precise automated regulation of the gas flow is important in a gaseous diffusion plant. These reasons make it necessary to equip the plant with a large number of special measuring, regulating and controlling systems.

UF6 is normally evaporated from cylinders located within autoclaves and is distributed in gaseous form to the entry point by way of cascade header pipework (collectors). The ‘product’ and ‘tails’ UF6 gaseous streams flowing from exit points are passed by way of cascade header pipework to either cold traps or to compression stations where the UF6 gas is liquified prior to onward transfer into suitable containers for transportation or storage. Because a gaseous diffusion enrichment plant consists of a large number of gaseous diffusion assemblies arranged in cascades, there are many kilometres of cascade header pipework (collectors) containing thousands of welds with much repetition of layout. Equipment, components and piping systems are manufactured such that they meet the very high vacuum and cleanliness standards.

5.4.1  Feed systems/product and tails withdrawal systems

Specially designed or prepared process systems, capable of operating at pressures of 300 kPa (45 psi) or less, including:

a) autoclaves (or systems), used for passing UF6 to the gaseous diffusion cascades;

b) desublimers (or cold traps) used to remove UF6 from diffusion cascades;

c) liquefaction stations where UF6 gas from the cascade is compressed and cooled to form liquid UF6;

d) ‘product’ or ‘tails’ stations used for transferring UF6 into containers.

    - Header piping systems (collectors)

Specially designed or prepared piping systems and header systems (collectors) for handling UF6 within the gaseous diffusion cascades. This piping network is normally of the ‘double’ header system (collectors) with each cell connected to each of the headers (collectors).

    - Vacuum systems

(a) Specially designed or prepared large vacuum manifolds, vacuum headers and vacuum pumps having a suction capacity of 5 m3/min (175 ft3/min) or more.

(b) Vacuum pumps specially designed for service in UF6-bearing atmospheres made of, or lined with, aluminium, nickel, or alloys bearing more than 60% nickel. These pumps may be either rotary or positive, may have displacement and fluorocarbon seals, and may have special working fluids present.

    - Special shut-off and control valves

Specially designed or prepared manual or automated shut-off and control bellows valves made of UF6-resistant materials with a diameter of 40 to 1,500 mm (1.5 to 59 in) for installation in main and auxiliary systems of gaseous diffusion enrichment plants.

    - UF6 mass spectrometers/ion sources

Specially designed or prepared magnetic or quadrupole mass spectrometers capable of taking ‘on-line’ samples of feed, product or tails, from UF6 gas streams and having all of the following characteristics:

- unit resolution for atomic mass unit greater than 320;

- ion sources constructed of or lined with nichrome or monel or nickel plated;

- electron bombardment ionisation sources;

- collector system suitable for isotopic analysis.

Explanatory note

The items listed above either come into direct contact with the UF6 process gas or directly control the flow within the cascade. All surfaces which come into contact with the process gas are wholly made of, or lined with, UF6-resistant materials. For the purposes of the sections relating to gaseous diffusion items, the materials resistant to corrosion by UF6 include stainless steel, aluminium, aluminium alloys, aluminium oxide, nickel or alloys containing 60% or more nickel and UF6-resistant fully fluorinated hydrocarbon polymers.

5.5.  Specially designed or prepared systems, equipment and components for use in aerodynamic enrichment plants

Introductory note

In aerodynamic enrichment processes, a mixture of gaseous UF6 and light gas (hydrogen or helium) is compressed and then passed through separating elements wherein isotopic separation is accomplished by the generation of high centrifugal forces over a curved-wall geometry. Two processes of this type have been successfully developed: the separation nozzle process and the vortex tube process. For both processes the main components of a separation stage include cylindrical vessels housing the special separation elements (nozzles or vortex tubes), gas compressors and heat exchangers to remove the heat of compression. An aerodynamic plant requires a number of these stages, so that quantities can provide an important indication of end use. Since aerodynamic processes use UF6, all equipment, pipeline and instrumentation surfaces (that come in contact with the gas) must be made of materials that remain stable in contact with UF6.

Explanatory note

The items listed in this section either come into direct contact with the UF6 process gas or directly control the flow within the cascade. All surfaces which come into contact with the process gas are wholly made of or protected by UF6-resistant materials. For the purposes of the section relating to aerodynamic enrichment items, the materials resistant to corrosion by UF6 include copper, stainless steel, aluminium, aluminium alloys, nickel or alloys containing 60% or more nickel and UF6-resistant fully fluorinated hydrocarbon polymers.

    - Separation nozzles

Specially designed or prepared separation nozzles and assemblies thereof. The separation nozzles consist of slit-shaped, curved channels having a radius of curvature less than 1 mm (typically 0.1 to 0.05 mm), resistant to corrosion by UF6 and having a knife-edge within the nozzle that separates the gas flowing through the nozzle into two fractions.

    - Vortex tubes

Specially designed or prepared vortex tubes and assemblies thereof. The vortex tubes are cylindrical or tapered, made of or protected by materials resistant to corrosion by UF6, having a diameter of between 0.5 cm and 4 cm, a length to diameter ratio of 20:1 or less and with one or more tangential inlets. The tubes may be equipped with nozzle type appendages at either or both ends.

Explanatory note

The feed gas enters the vortex tube tangentially at one end or through swirl vanes or at numerous tangential positions along the periphery of the tube.

    - Compressors and gas blowers

Specially designed or prepared axial, centrifugal or positive displacement compressors or gas blowers made of or protected by materials resistant to corrosion by UF6 and with a suction volume capacity of 2 m3/min or more of UF6/carrier gas (hydrogen or helium) mixture.

Explanatory note

These compressors and gas blowers typically have a pressure ratio between 1.2:1 and 6:1.

    - Rotary shaft seals

Specially designed or prepared rotary shaft seals, with seal feed and seal exhaust connections, for sealing the shaft connecting the compressor rotor or the gas blower rotor with the driver motor so as to ensure a reliable seal against out-leakage of process gas or in-leakage of air or seal gas into the inner chamber of the compressor or gas blower which is filled with a UF6/carrier gas mixture.

    - Heat exchangers for gas cooling

Specially designed or prepared heat exchangers made of or protected by materials resistant to corrosion by UF6.

    - Separation element housings

Specially designed or prepared separation element housings, made of or protected by materials resistant to corrosion by UF6, for containing vortex tubes or separation nozzles.

Explanatory note

These housings may be cylindrical vessels greater than 300 mm in diameter and greater than 900 mm in length, or may be rectangular vessels of comparable dimensions, and may be designed for horizontal or vertical installation.

    - Feed systems/product and tails withdrawal systems

Specially designed or prepared process systems or equipment for enrichment plants made of or protected by materials resistant to corrosion by UF6, including:

- feed autoclaves, ovens, or systems used for passing UF6 to the enrichment process;

(b) desublimers (or cold traps) used to remove UF6 from the enrichment process for subsequent transfer upon heating;

(c) solidification or liquefaction stations used to remove UF6 from the enrichment process by compressing and converting UF6 to a liquid or solid form;

(d) 'product' or 'tails' stations used for transferring UF6 into containers.

5.5.8  Header piping systems (collectors)

Specially designed or prepared header piping systems (collectors), made of or protected by materials resistant to corrosion by UF6, for handling UF6 within the aerodynamic cascades. This piping network is normally of the 'double' header design with each stage or group of stages connected to each of the headers (collectors).

5.5.9  Vacuum systems and pumps

(a) Specially designed or prepared vacuum systems having a suction capacity of 5 m3/min or more, consisting of vacuum manifolds, vacuum headers (collectors) and vacuum pumps, and designed for service in UF6-bearing atmospheres.

(b) Vacuum pumps specially designed or prepared for service in UF6-bearing atmospheres and made of or protected by materials resistant to corrosion by UF6. These pumps may use fluorocarbon seals and special working fluids.

5.5.10  Special shut-off and control valves

Specially designed or prepared manual or automated shut-off and control bellows valves made of or protected by materials resistant to corrosion by UF6 with a diameter of 40 to 1,500 mm for installation in main and auxiliary systems of aerodynamic enrichment plants.

5.5.11  UF6 mass spectrometers/ion sources

Specially designed or prepared magnetic or quadrupole mass spectrometers capable of taking ‘on-line’ samples of feed, ‘product’ or ‘tails’, from UF6 gas streams and having all of the following characteristics:

1. unit resolution for mass greater than 320;

2. ion sources constructed of or lined with nichrome or monel or nickel plated;

3. electron bombardment ionisation sources;

4. collector system suitable for isotopic analysis.

    - UF6/carrier gas separation systems

Specially designed or prepared process systems for separating UF6 from carrier gas (hydrogen or helium).

Explanatory note

These systems are designed to reduce the UF6 content in the carrier gas to 1 ppm or less and may incorporate equipment such as:

(a) cryogenic heat exchangers and cryoseparators capable of temperatures of -120°C or less, or

(b) cryogenic refrigeration units capable of temperatures of -120°C or less, or

(c) separation nozzle or vortex tube units for the separation of UF6 from carrier gas, or

(d) UF6 cold traps capable of temperatures of -20°C or less.

5.6.  Specially designed or prepared systems, equipment and components for use in chemical exchange or ion exchange enrichment plants

Introductory note

The slight difference in mass between the isotopes of uranium causes small changes in chemical reaction equilibria that can be used as a basis for separation of the isotopes. Two processes have been successfully developed: liquid-liquid chemical exchange and solid-liquid ion exchange.

In the liquid-liquid chemical exchange process, immiscible liquid phases (aqueous and organic) are countercurrently contacted to give the cascading effect of thousands of separation stages. The aqueous phase consists of uranium chloride in hydrochloric acid solution; the organic phase consists of an extractant containing uranium chloride in an organic solvent. The contactors employed in the separation cascade can be liquid-liquid exchange columns (such as pulsed columns with sieve plates) or liquid centrifugal contactors. Chemical conversions (oxidation and reduction) are required at both ends of the separation cascade in order to provide for the reflux requirements at each end. A major design concern is to avoid contamination of the process streams with certain metal ions. Plastic, plastic-lined (including use of fluorocarbon polymers) and/or glass-lined columns and piping are therefore used.

In the solid-liquid ion-exchange process, enrichment is accomplished by uranium adsorption/desorption on a special, very fast-acting, ion-exchange resin or adsorbent. A solution of uranium in hydrochloric acid and other chemical agents is passed through cylindrical enrichment columns containing packed beds of the adsorbent. For a continuous process, a reflux system is necessary to release the uranium from the adsorbent back into the liquid flow so that 'product' and 'tails' can be collected. This is accomplished with the use of suitable reduction/oxidation chemical agents that are fully regenerated in separate external circuits and that may be partially regenerated within the isotopic separation columns themselves. The presence of hot concentrated hydrochloric acid solutions in the process requires that the equipment be made of or protected by special corrosion-resistant materials.

5.6.1  Liquid-liquid exchange columns (chemical exchange)

Countercurrent liquid-liquid exchange columns having mechanical power input (i.e., pulsed columns with sieve plates, reciprocating plate columns, and columns with internal turbine mixers), specially designed or prepared for uranium enrichment using the chemical exchange process. For corrosion resistance to concentrated hydrochloric acid solutions, these columns and their internals are made of or protected by suitable plastic materials (such as fluorocarbon polymers) or glass. The stage residence time of the columns is designed to be short (30 seconds or less).

5.6.2  Liquid-liquid centrifugal contactors (chemical exchange)

Liquid-liquid centrifugal contactors specially designed or prepared for uranium enrichment using the chemical exchange process. Such contactors use rotation to achieve dispersion of the organic and aqueous streams and then centrifugal force to separate the phases. For corrosion resistance to concentrated hydrochloric acid solutions, the contactors are made of or are lined with suitable plastic materials (such as fluorocarbon polymers) or are lined with glass. The stage residence time of the centrifugal contactors is designed to be short (30 seconds or less).

    - Uranium reduction systems and equipment (chemical exchange)

(a) Specially designed or prepared electrochemical reduction cells to reduce uranium from one valence state to another for uranium enrichment using the chemical exchange process. The cell materials in contact with process solutions must be corrosion resistant to concentrated hydrochloric acid solutions.

Explanatory note

The cell cathodic compartment must be designed to prevent re-oxidation of uranium to its higher valence state. To keep the uranium in the cathodic compartment, the cell may have an impervious diaphragm membrane constructed of special cation exchange material. The cathode consists of a suitable solid conductor such as graphite.

(b) Specially designed or prepared systems at the product end of the cascade for taking the U+4 out of the organic stream, adjusting the acid concentration and feeding to the electrochemical reduction cells.

Explanatory note

These systems consist of solvent extraction equipment for stripping the U+4 from the organic stream into an aqueous solution, evaporation and/or other equipment to accomplish solution pH adjustment and control, and pumps or other transfer devices for feeding to the electrochemical reduction cells. A major design concern is to avoid contamination of the aqueous stream with certain metal ions. Consequently, for those parts in contact with the process stream, the system is constructed of equipment made of or protected by suitable materials (such as glass, fluorocarbon polymers, polyphenyl sulphate, polyether sulphone, and resin-impregnated graphite).

    - Feed preparation systems (chemical exchange)

Specially designed or prepared systems for producing high-purity uranium chloride feed solutions for chemical exchange uranium isotope separation plants.

Explanatory note

These systems consist of dissolution, solvent extraction and/or ion exchange equipment for purification and electrolytic cells for reducing the uranium U+6 or U+4 to U+3. These systems produce uranium chloride solutions having only a few parts per million of metallic impurities such as chromium, iron, vanadium, molybdenum and other bivalent or higher multi-valent cations. Materials of construction for portions of the system processing high-purity U+3 include glass, fluorocarbon polymers, polyphenyl sulphate or polyether sulphone plastic-lined and resin-impregnated graphite.

    - Uranium oxidation systems (chemical exchange)

Specially designed or prepared systems for oxidation of U+3 to U+4 for return to the uranium isotope separation cascade in the chemical exchange enrichment process.

Explanatory note

These systems may incorporate equipment such as:

(a) Equipment for contacting chlorine and oxygen with the aqueous effluent from the isotope separation equipment and extracting the resultant U+4 into the stripped organic stream returning from the product end of the cascade.

(b) Equipment that separates water from hydrochloric acid so that the water and the concentrated hydrochloric acid may be reintroduced to the process at the proper locations.

    - Fast-reacting ion exchange resins/adsorbents (ion exchange)

Fast-reacting ion-exchange resins or adsorbents specially designed or prepared for uranium enrichment using the ion exchange process, including porous macro-reticular resins, and/or pellicular structures in which the active chemical exchange groups are limited to a coating on the surface of an inactive porous support structure, and other composite structures in any suitable form including particles or fibres. These ion exchange resins/adsorbents have diameters of 0.2 mm or less and must be chemically resistant to concentrated hydrochloric acid solutions as well as physically strong enough so as not to degrade in the exchange columns. The resins/adsorbents are specially designed to achieve very fast uranium isotope exchange kinetics (exchange rate half-time of less than 10 seconds) and are capable of operating at a temperature in the range of 100°C to 200°C.

    - Ion exchange columns (ion exchange)

Cylindrical columns greater than 1,000 mm in diameter for containing and supporting packed beds of ion exchange resin/adsorbent, specially designed or prepared for uranium enrichment using the ion exchange process. These columns are made of or protected by materials (such as titanium or fluorocarbon plastics) resistant to corrosion by concentrated hydrochloric acid solutions and are capable of operating at a temperature in the range of 100°C to 200°C and pressures above 0.7 MPa (102 psi).

    - Ion exchange reflux systems (ion exchange)

(a) Specially designed or prepared chemical or electrochemical reduction systems for regeneration of the chemical reducing agent(s) used in ion exchange uranium enrichment cascades.

(b) Specially designed or prepared chemical or electrochemical oxidation systems for regeneration of the chemical oxidising agent(s) used in ion exchange uranium enrichment cascades.

Explanatory note

The ion exchange enrichment process may use, for example, trivalent titanium (Ti+3) as a reducing cation, in which case the reduction system would regenerate Ti+3 by reducing Ti+4. The process may use, for example, trivalent iron (Fe+3) as an oxidant, in which case the oxidation system would regenerate Fe+3 by oxidising Fe+2.

5.7. Specially designed or prepared systems, equipment and components for use in laser-based enrichment plants

Introductory note

Present systems for enrichment processes using lasers fall into two categories: those in which the process medium is atomic uranium vapour and those in which the process medium is the vapour of a uranium compound. Common nomenclature for such processes include: first category - atomic vapour laser isotope separation (AVLIS or SILVA); second category - molecular laser isotope separation (MLIS or MOLIS) and chemical reaction by isotope selective laser activation (CRISLA). The systems, equipment and components for laser enrichment plants embrace: (a) devices to feed uranium-metal vapour (for selective photo-ionisation) or devices to feed the vapour of a uranium compound (for photo-dissociation or chemical activation); (b) devices to collect enriched and depleted uranium metal as 'product' and 'tails' in the first category, and devices to collect dissociated or reacted compounds as 'product' and unaffected material as 'tails' in the second category; (c) process laser systems to selectively excite the uranium-235 species; and (d) feed preparation and product conversion equipment. The complexity of the spectroscopy of uranium atoms and compounds may require incorporation of any of a number of available laser technologies.

Explanatory note

Many of the items listed in this section come into direct contact with uranium metal vapour or liquid or with process gas consisting of UF6 or a mixture of UF6 and other gases. All surfaces that come into contact with the uranium or UF6 are wholly made of or protected by corrosion-resistant materials. For the purposes of the section relating to laser-based enrichment items, the materials resistant to corrosion by the vapour or liquid of uranium metal or uranium alloys include yttria-coated graphite and tantalum; and the materials resistant to corrosion by UF6 include copper, stainless steel, aluminium, aluminium alloys, nickel or alloys containing 60% or more nickel and UF6-resistant fully fluorinated hydrocarbon polymers.

5.7.1  Uranium vaporisation systems (AVLIS)

Specially designed or prepared uranium vaporisation systems which contain high power strip or scanning electron beam guns with a delivered power on the target of more than 2.5 kW/cm.

    - Liquid uranium metal handling systems (AVLIS)

Specially designed or prepared liquid metal handling systems for molten uranium or uranium alloys, consisting of crucibles and cooling equipment for the crucibles.

Explanatory note

The crucibles and other parts of this system that come into contact with molten uranium or uranium alloys are made of or protected by materials of suitable corrosion and heat resistance. Suitable materials include tantalum, yttria-coated graphite, graphite coated with other rare earth oxides (see Annex to the Regulation laying down a list of selected dual-use items in the nuclear sector) or mixtures thereof.

5.7.3  Uranium metal ‘product’ and ‘tails’ collector assemblies (AVLIS)

Specially designed or prepared ‘product’ and ‘tails’ collector assemblies for uranium metal in liquid or solid form.

Explanatory note

Components for these assemblies are made of or protected by materials resistant to the heat and corrosion of uranium metal vapour or liquid (such as yttria-coated graphite or tantalum) and may include pipes, valves, fittings, ‘gutters’, feed-throughs, heat exchangers and collector plates for magnetic, electrostatic or other separation methods.

    - Separator module housings (AVLIS)

Specially designed or prepared cylindrical or rectangular vessels for containing the uranium metal vapour source, the electron beam gun, and the ‘product’ and ‘tails’ collectors.

Explanatory note

These housings have multiplicity of ports for electrical and water feed-throughs, laser beam windows, vacuum pump connections and instrumentation diagnostics and monitoring. They have provisions for opening and closure to allow refurbishment of internal components.

    - Supersonic expansion nozzles (MLIS)

Specially designed or prepared supersonic expansion nozzles for cooling mixtures of UF6 and carrier gas to 150 K or less and which are corrosion resistant to UF6.

    - Uranium pentafluoride product collectors (MLIS)

Specially designed or prepared uranium pentafluoride (UF5) solid product collectors consisting of filter, impact, or cyclone-type collectors, or combinations thereof, and which are corrosion resistant to the UF5/UF6 environment.

    - UF6/carrier gas compressors (MLIS)

Specially designed or prepared compressors for UF6/carrier gas mixtures, designed for long term operation in a UF6 environment. The components of these compressors that come into contact with process gas are made of or protected by materials resistant to corrosion by UF6.

    - Rotary shaft seals (MLIS)

Specially designed or prepared rotary shaft seals, with seal feed and seal exhaust connections, for sealing the shaft connecting the compressor rotor with the driver motor so as to ensure a reliable seal against out-leakage of process gas or in-leakage of air or seal gas into the inner chamber of the compressor which is filled with a UF6/carrier gas mixture.

    - Fluorination systems (MLIS)

Specially designed or prepared systems for fluorinating UF5 (solid) to UF6 (gas).

Explanatory note

These systems are designed to fluorinate the collected UF5 powder to UF6 for subsequent collection in product containers or for transfer as feed to MLIS units for additional enrichment. In one approach, the fluorination reaction may be accomplished within the isotope separation system to react and recover directly off the 'product' collectors. In another approach, the UF5 powder may be removed/transferred from the 'product' collectors into a suitable reaction vessel (e.g., fluidised-bed reactor, screw reactor or flame tower) for fluorination. In both approaches, equipment for storage and transfer of fluorine (or other suitable fluorinating agents) and for collection and transfer of UF6 are used.

    - UF6 mass spectrometers/ion sources (MLIS)

Specially designed or prepared magnetic or quadrupole mass spectrometers capable of taking ‘on-line’ samples of feed, ‘product’ or ‘tails’, from UF6 gas streams and having all of the following characteristics:

1. unit resolution for mass greater than 320;

2. ion sources constructed of or lined with nichrome or monel or nickel plated;

3. electron bombardment ionisation sources;

4. collector system suitable for isotopic analysis.

    - Feed systems/product and tails withdrawal systems (MLIS)

Specially designed or prepared process systems or equipment for enrichment plants made of or protected by materials resistant to corrosion by UF6, including:

- Feed autoclaves, ovens, or systems used for passing UF6 to the enrichment process.

- Desublimers (or cold traps) used to remove UF6 from the enrichment process for subsequent transfer upon heating.

- Solidification or liquefaction stations used to remove UF6 from the enrichment process by compressing and converting UF6 to a liquid or solid form.

(d) ‘Product’ or ‘tails’ stations used for transferring UF6 into containers.

    - UF6/carrier gas separation systems (MLIS)

Specially designed or prepared process systems for separating UF6 from carrier gas. The carrier gas may be nitrogen, argon, or other gas.

Explanatory note

These systems may incorporate equipment such as:

(a) cryogenic heat exchangers or cryoseparators capable of temperatures of -120°C or less, or

(b) cryogenic refrigeration units capable of temperatures of -120°C or less, or

(c) UF6 cold traps capable of temperatures of -20°C or less.

    - Laser systems (AVLIS, MLIS and CRISLA)

Lasers or laser systems specially designed or prepared for the separation of uranium isotopes.

Explanatory note

The lasers and laser components of importance in laser-based enrichment processes include those identified in INFCIRC/254/Part 2 - (as amended). The laser system for the AVLIS process usually consists of two lasers: a copper vapour laser and a dye laser. The laser system for MLIS usually consists of a CO2 or excimer laser and a multi-pass optical cell with revolving mirrors at both ends. Lasers or laser systems for both processes require a spectrum frequency stabiliser for operation over extended periods of time.

5.8.  Specially designed or prepared systems, equipment and components for use in plasma separation enrichment plants

###### Introductory note

In the plasma separation process, a plasma of uranium ions passes through an electric field tuned to the 235U ion resonance frequency so that they preferentially absorb energy and increase the diameter of their corkscrew-like orbits. Ions with a large diameter path are trapped to produce a product enriched in 235U. The plasma, which is made by ionising uranium vapour, is contained in a vacuum chamber with a high strength magnetic field produced by a superconducting magnet. The main technological systems of the process include the uranium plasma generation system, the separator module with superconducting magnet (see Annex to the Regulation laying down a list of selected dual-use items in the nuclear sector), and metal removal systems for the collection of ‘product’ and ‘tails’.

5.8.1  Microwave power sources and antennae

Specially designed or prepared microwave power sources and antennae for producing or accelerating ions and having the following characteristics: greater than 30 GHz frequency and greater than 50 kW mean power output for ion production.

5.8.2  Ion excitation coils

Specially designed or prepared radio frequency ion excitation coils for frequencies of more than 100 kHz and capable of handling more than 40 kW mean power.

5.8.3  Uranium plasma generation systems

Specially designed or prepared systems for the generation of uranium plasma, which may contain high-power strip or scanning electron beam guns with a delivered power on the target of more than 2.5 kW/cm.

5.8.4  Liquid uranium metal handling systems

Specially designed or prepared liquid metal handling systems for molten uranium or uranium alloys, consisting of crucibles and cooling equipment for the crucibles.

Explanatory note

The crucibles and other parts of this system that come into contact with molten uranium or uranium alloys are made of or protected by materials of suitable corrosion and heat resistance. Suitable materials include tantalum, yttria-coated graphite, graphite coated with other rare earth oxides (see Annex to the Regulation laying down a list of selected dual-use items in the nuclear sector) or mixtures thereof.

5.8.5  Uranium metal ‘product’ and ‘tails’ collector assemblies

Specially designed or prepared ‘product’ and ‘tails’ collector assemblies for uranium metal in solid form. These collector assemblies are made of or protected by materials resistant to the heat and corrosion of uranium metal vapour, such as yttria-coated graphite or tantalum.

5.8.6  Separator module housings

Cylindrical vessels specially designed or prepared for use in plasma separation enrichment plants for containing the uranium plasma source, radio-frequency drive coil and the ‘product’ and ‘tails’ collectors.

Explanatory note

These housings have a multiplicity of ports for electrical feed-throughs, diffusion pump connections and instrumentation diagnostics and monitoring. They have provisions for opening and closure to allow for refurbishment of internal components and are constructed of a suitable non-magnetic material such as stainless steel.

5.9.  Specially designed or prepared systems, equipment and components for use in electromagnetic enrichment plants

Introductory note

In the electromagnetic process, uranium metal ions produced by ionisation of a salt feed material (typically UCl4) are accelerated and passed through a magnetic field that has the effect of causing the ions of different isotopes to follow different paths. The major components of an electromagnetic isotope separator include: a magnetic field for ion-beam diversion/separation of the isotopes, an ion source with its acceleration system, and a collection system for the separated ions. Auxiliary systems for the process include the magnet power supply system, the ion source high-voltage power supply system, the vacuum system, and extensive chemical handling systems for recovery of product and cleaning/recycling of components.

5.9.1  Electromagnetic isotope separators

Electromagnetic isotope separators specially designed or prepared for the separation of uranium isotopes, and equipment and components therefor, including:

- Ion sources

Specially designed or prepared single or multiple uranium ion sources consisting of a vapour source, ioniser, and beam accelerator, constructed of suitable materials such as graphite, stainless steel, or copper, and capable of providing a total ion beam current of 50 mA or greater.

- Ion collectors

Collector plates consisting of two or more slits and pockets specially designed or prepared for collection of enriched and depleted uranium ion beams and constructed of suitable materials such as graphite or stainless steel.

- Vacuum housings

Specially designed or prepared vacuum housings for uranium electromagnetic separators, constructed of suitable non-magnetic materials such as stainless steel and designed for operation at pressures of 0.1 Pa or lower.

Explanatory note

The housings are specially designed to contain the ion sources, collector plates and water-cooled liners and have provision for diffusion pump connections and opening and closure for removal and reinstallation of these components.

- Magnet pole pieces

Specially designed or prepared magnet pole pieces having a diameter greater than 2 m used to maintain a constant magnetic field within an electromagnetic isotope separator and to transfer the magnetic field between adjoining separators.

5.9.2  High voltage power supplies

Specially designed or prepared high-voltage power supplies for ion sources, having all of the following characteristics: capable of continuous operation, output voltage of 20 kV or greater, output current of 1 A or greater, and voltage regulation of better than 0.01% over a time period of 8 hours.

5.9.3  Magnet power supplies

Specially designed or prepared high-power, direct current magnet power supplies having all of the following characteristics: capable of continuously producing a current output of 500 A or greater at a voltage of 100 V or greater and with a current or voltage regulation better than 0.01% over a period of 8 hours.

6.	Plants for the manufacture and reworking of concentrations of heavy water, deuterium and its compounds and devices specially designed 	or prepared therefor

#### Introductory note

Heavy water can be produced by a variety of processes. However, the two processes that have proven to be commercially viable are the water-hydrogen sulphide exchange process (GS process) and the ammonia-hydrogen exchange process.

The GS process is based upon the exchange of hydrogen and deuterium between water and hydrogen sulphide within a series of towers which are operated with the top section cold and the bottom section hot. Water flows down the towers while the hydrogen sulphide gas circulates from the bottom to the top of the towers. A series of perforated trays are used to promote mixing between the gas and the water. Deuterium migrates to the water at low temperatures and to the hydrogen sulphide at high temperatures. Gas or water, enriched in deuterium, is removed from the first stage towers at the junction of the hot and cold sections and the process is repeated in subsequent stage towers. The product of the last stage, water enriched up to 30% in deuterium, is sent to a distillation unit to produce reactor grade heavy water; i.e., 99.75% deuterium oxide.

The ammonia-hydrogen exchange process can extract deuterium from synthesis gas through contact with liquid ammonia in the presence of a catalyst. The synthesis gas is fed into exchange towers and to an ammonia converter. Inside the towers the gas flows from the bottom to the top while the liquid ammonia flows from the top to the bottom. The deuterium is stripped from the hydrogen in the synthesis gas and concentrated in the ammonia. The ammonia then flows into an ammonia cracker at the bottom of the tower while the gas flows into an ammonia converter at the top. Further enrichment takes place in subsequent stages and reactor grade heavy water is produced through final distillation. The synthesis gas feed can be provided by an ammonia plant that, in turn, can be constructed in association with a heavy water ammonia-hydrogen exchange plant. The ammonia-hydrogen exchange process can also use ordinary water as a feed source of deuterium.

Many of the key equipment items for heavy water production plants using GS or the ammonia-hydrogen exchange processes are common to several segments of the chemical and petroleum industries. This is particularly so for small plants using the GS process. However, few of the items are available ‘off-the-shelf’. The GS and ammonia-hydrogen processes require the handling of large quantities of flammable, corrosive and toxic fluids at elevated pressures. Accordingly, in establishing the design and operating standards for plants and equipment using these processes, careful attention to the materials selection and specifications is required to ensure long service life with high safety and reliability factors. The choice of scale is primarily a function of economics and need. Thus, most of the equipment items would be prepared according to the requirements of the customer.

Finally, it should be noted that, in both the GS and the ammonia-hydrogen exchange processes, items of equipment which individually are not specially designed or prepared for heavy water production can be assembled into systems which are specially designed or prepared for producing heavy water. The catalyst production system used in the ammonia-hydrogen exchange process and water distillation systems used for the final concentration of heavy water to reactor-grade in either process are examples of such systems.

The items of equipment which are specially designed or prepared for the production of heavy water utilising either the water-hydrogen sulphide exchange process or the ammonia-hydrogen exchange process include the following:

  - Water-hydrogen sulphide exchange towers

Exchange towers fabricated from fine carbon steel (such as ASTM A516) with diameters of 6-9 m (20-30 ft), capable of operating at pressures greater than or equal to 2 MPa (300 psi) and with a corrosion allowance of 6 mm or greater, specially designed or prepared for heavy water production utilising the water hydrogen sulphide exchange process.

  - Blowers and compressors

Single stage, low head (i.e., 0.2 MPa or 30 psi) centrifugal blowers or compressors for hydrogen-sulphide gas circulation (i.e., gas containing more than 70% H2S) specially designed or prepared for heavy water production utilising the water-hydrogen sulphide exchange process. These blowers or compressors have a throughput capacity greater than or equal to 56 m3/second (120,000 SCFM) while operating at pressures greater than or equal to 1.8 MPa (260 psi) suction and have seals designed for wet H2S service.

  - Ammonia-hydrogen exchange towers

Ammonia-hydrogen exchange towers greater than or equal to 35 m (114.3 ft) in height with diameters of 1.5-2.5 m (4.9-8.2 ft) capable of operating at pressures greater than 15 MPa (2,225 psi) specially designed or prepared for heavy water production utilising the ammonia-hydrogen exchange process. These towers also have at least one flanged, axial opening of the same diameter as the cylindrical part through which the tower internals can be inserted or withdrawn.

  - Tower internals and stage pumps

Tower internals and stage pumps specially designed or prepared for towers for heavy water production utilising the ammonia-hydrogen exchange process. Tower internals include specially designed stage contactors which promote intimate gas/liquid contact. Stage pumps include specially designed submersible pumps for circulation of liquid ammonia within a contacting stage internal to the stage towers.

  - Ammonia crackers

Ammonia crackers with operating pressures greater than or equal to 3 MPa (450 psi) specially designed or prepared for heavy water production utilising the ammonia hydrogen exchange process.

  - Infrared absorption analysers

Infrared absorption analysers capable of ‘on-line’ hydrogen/deuterium ratio analysis where deuterium concentrations are equal to or greater than 90%.

6.7.  Catalytic burners Catalytic burners for the conversion of enriched deuterium gas into heavy water specially designed or prepared for heavy water production utilising the ammonia hydrogen exchange process.

  - Complete heavy water upgrade systems or columns therefor

Complete heavy water upgrade systems, or columns therefor, specially designed or prepared for the upgrade of heavy water to reactor-grade deuterium concentration.

###### Explanatory note

These systems, which usually employ water distillation to separate heavy water from light water, are specially designed or prepared to produce reactor-grade heavy water (i.e., typically 99.75% deuterium oxide) from heavy water feedstock of lesser concentration.

7.	Plants for the conversion of uranium and plutonium for use in the manufacture of fuel elements and the separation of the uranium isotopes specified in Parts 4 and 5, and equipment specially designed or prepared therefor

EXPORTS

All of the plants, systems, and specially designed or prepared equipment within this boundary can be used for the processing, production, or use of special fissionable material. The export of the whole set of major items within this boundary will take place only in accordance with the procedures set out in Part 8.

7.1.  Plants for the conversion of uranium and equipment specially designed or prepared therefor

Introductory note

Uranium conversion plants and systems may perform one or more transformations from one uranium chemical species to another, including: conversion of uranium ore concentrates to UO3, conversion of UO3 to UO2, conversion of uranium oxides to UF4, UF6, or UCl4, conversion of UF4 to UF6, conversion of UF6 to UF4, conversion of UF4 to uranium metal, and conversion of uranium fluorides to UO2. Many of the key equipment items for uranium conversion plants are common to several segments of the chemical process industry. For example, the types of equipment employed in these processes may include: furnaces, rotary kilns, fluidised bed reactors, flame tower reactors, liquid centrifuges, distillation columns and liquid-liquid extraction columns. However, few of the items are available ‘off-the-shelf’; most would be prepared according to the requirements and specifications of the customer. In some instances, special design and construction considerations are required to address the corrosive properties of some of the chemicals handled (HF, F2, CIF3, and uranium fluorides), as well as nuclear criticality concerns. Finally, it should be noted that, in all of the uranium conversion processes, items of equipment which individually are not specially designed or prepared for uranium conversion can be assembled into systems which are specially designed or prepared for use in uranium conversion.

7.1.1  Specially designed or prepared systems for the conversion of uranium ore concentrates to UO3

Explanatory note

Conversion of uranium ore concentrates to UO3 can be performed by first dissolving the ore in nitric acid and extracting purified uranyl nitrate using a solvent such as tributyl phosphate. Next, the uranyl nitrate is converted to UO3 either by concentration and denitration or by neutralisation with gaseous ammonia to produce ammonium diuranate with subsequent filtering, drying, and calcining.

7.1.2  Specially designed or prepared systems for the conversion of UO3 to UF6

###### Explanatory note

Conversion of UO3 to UF6 can be performed directly by fluorination. The process requires a source of fluorine gas or chlorine trifluoride.

7.1.3  Specially designed or prepared systems for the conversion of UO3 to UO2

Explanatory note

Conversion of UO3 to UO2 can be performed through reduction of UO3 with cracked ammonia gas or hydrogen.

7.1.4  Specially designed or prepared systems for the conversion of UO2 to UF4

Explanatory note

Conversion of UO2 to UF4 can be performed by reacting UO2 with hydrogen fluoride gas (HF) at 300-500°C.

7.1.5  Specially designed or prepared systems for the conversion of UF4 to UF6

Explanatory note

Conversion of UF4 to UF6 is performed by exothermic reaction with fluorine in a tower reactor. UF6 is condensed from the hot effluent gases by passing the effluent stream through a cold trap cooled to -10°C. The process requires a source of fluorine gas.

7.1.6  Specially designed or prepared systems for the conversion of UF4 to U metal

Explanatory note

Conversion of UF4 to U metal is performed by reduction with magnesium (large batches) or calcium (small batches). The reaction is carried out at temperatures above the melting point of uranium (1,130 °C).

7.1.7  Specially designed or prepared systems for the conversion of UF6 to UO2

Explanatory note

Conversion of UF6 to UO2 can be performed by one of three processes. In the first, UF6 is reduced and hydrolysed to UO2 using hydrogen and steam. In the second, UF6 is hydrolysed by solution in water, ammonia is added to precipitate ammonium diuranate, and the diuranate is reduced to UO2 with hydrogen at 820°C. In the third process, gaseous UF6, CO2, and NH3 are combined in water, precipitating ammonium uranyl carbonate. The ammonium uranyl carbonate is combined with steam and hydrogen at 500-600°C to yield UO2.

UF6 to UO2 conversion is often performed as the first stage of a fuel fabrication plant.

7.1.8  Specially designed or prepared systems for the conversion of UF6 to UF4

Explanatory note

Conversion of UF6 to UF4 is performed by reduction with hydrogen.

7.1.9  Specially designed or prepared systems for the conversion of UO2 to UCl4

###### Explanatory note

Conversion of UO2 to UCl4 can be performed by one of two processes. In the first, UO2 is reacted with carbon tetrachloride (CCl4 ) at approximately 400ºC. In the second, UO2 is reacted at approximately 700ºC in the presence of carbon black (CAS 1333-86-4), carbon monoxide, and chlorine to yield UCl4.

7.2.  Plants for the conversion of plutonium and equipment specially designed or prepared therefor

Introductory note

Plutonium conversion plants and systems perform one or more transformations from one plutonium chemical species to another, including: conversion of plutonium nitrate to PuO2, conversion of PuO2 to PuF4, and conversion of PuF4 to plutonium metal. Plutonium conversion plants are usually associated with reprocessing facilities, but may also be associated with plutonium fuel fabrication facilities. Many of the key equipment items for plutonium conversion plants are common to several segments of the chemical process industry. For example, the types of equipment employed in these processes may include: furnaces, rotary kilns, fluidised bed reactors, flame tower reactors, liquid centrifuges, distillation columns and liquid-liquid extraction columns. Hot chambers, glove boxes and remote manipulators may also be required. However, few of the items are available ‘off-the-shelf’; most would be prepared according to the requirements and specifications of the customer. Particular care in designing for the special radiological, toxicity and criticality hazards associated with plutonium is essential. In some instances, special design and construction considerations are required to address the corrosive properties of some of the chemicals handled (e.g. HF). Finally, it should be noted that, for all plutonium conversion processes, items of equipment which individually are not specially designed or prepared for plutonium conversion can be assembled into systems which are specially designed or prepared for use in plutonium conversion.

7.2.1  Specially designed or prepared systems for the conversion of plutonium nitrate to oxide

Explanatory note

The main functions involved in this process are: process feed storage and adjustment, precipitation and solid/liquor separation, calcination, product handling, ventilation, waste management, and process control. The process systems are particularly adapted so as to avoid criticality and radiation effects and to minimise toxicity hazards. In most reprocessing facilities, this process involves the conversion of plutonium nitrate to plutonium dioxide. Other processes can involve the precipitation of plutonium oxalate or plutonium peroxide.

7.2.2  Specially designed or prepared systems for plutonium metal production

Explanatory note

This process usually involves the fluorination of plutonium dioxide, normally with highly corrosive hydrogen fluoride, to produce plutonium fluoride which is subsequently reduced using high purity calcium metal to produce metallic plutonium and a calcium fluoride slag. The main functions involved in this process are fluorination (e.g. involving equipment fabricated or lined with a precious metal), metal reduction (e.g. employing ceramic crucibles), slag recovery, product handling, ventilation, waste management and process control. The process systems are particularly adapted so as to avoid criticality and radiation effects and to minimise toxicity hazards. Other processes include the fluorination of plutonium oxalate or plutonium peroxide followed by a reduction to metal.

8.	General notes, technical controls and definitions

8.1.  General comments

(a) Non-proliferation of nuclear weapons

The export of the items and associated technologies listed in this Regulation should only be permitted on the basis of government guarantees from the receiving country that expressly rule out any use that could lead to the construction of any type of nuclear explosive device. The Office will request a government guarantee.

(b) Provision of a guarantee

The export of the items and associated technologies listed in this Regulation to a state that does not own nuclear weapons should be allowed only if the receiving country has entered into an agreement with the IAEA on the provision of a guarantee in respect of all source and separate fissionable materials for present and future peaceful activities. Exportation to a state that does not own nuclear weapons and that has not entered into such an agreement should only be allowed under exceptional circumstances where it is considered necessary for the safe operation of existing plants, and if the guarantees are applied to these plants.

(c) Physical protection

All items and associated technologies listed in this Regulation should come under effective physical protection in order to prevent their unauthorised use and handling. The level of physical protection to be provided in relation to the type of materials, equipment and plants is stipulated in the implementing legislation.

(d) Control of subsequent transfers

In the case of the subsequent export of the items and associated technologies listed in this Regulation or the export of selected items gained from plants originally exported by the supplier or using equipment or technologies originally exported by the supplier, the state that is receiving the subsequent exportation will provide the same guarantees that were demanded by the supplier of the original export.

(e) If the IAEA announces that a recipient state is violating its obligations in honouring its agreement on guarantees, the suppliers should consider suspending the export of the items and associated technologies listed in this Regulation to that state during the period of investigation by the IAEA. Suspension of exports of the selected items to states that violate their obligations in the field of nuclear non-proliferation and guarantees is a matter for national governments or the UN Security Council to decide on.

(f) Export of components of controlled items

These inspections should not be circumvented by exporting components of controlled items.

8.2.  Technological control

(a) The export of ‘technology’ directly associated with any item in the list will be subject to as great a degree of scrutiny and control as will the item itself, to the extent permitted by national legislation.

(b) Controls on ‘technology’ transfer do not apply to information ‘in the public domain’ or to

‘basic scientific research’.

  - Definitions

‘Technology’ means specific information required for the ‘development’, ‘production’, or ‘use’ of any item contained in the list. This information may take the form of ‘technical data’, or ‘technical assistance’.

‘Basic scientific research’ is experimental or theoretical work undertaken principally to acquire new knowledge of the fundamental principles of phenomena and observable facts, not primarily directed towards a specific practical aim or objective.

‘Development’ is related to all phases before ‘production’ such as:

- design

- design research

- design analysis

- design concepts

- assembly and testing of prototypes

- pilot production schemes

- design data

- process of transforming design data into a product

- configuration design

- integration design

- layouts

‘In the public domain’, as it applies herein, means technology that has been made available without restrictions upon its further dissemination. (Copyright restrictions do not remove technology from being in the public domain.)

‘Production’ means all production phases such as:

- construction

- production engineering

- manufacture

- integration

- assembly (mounting)

- inspection

- testing

- quality assurance

‘Technical assistance’ may take forms such as: instruction, skills, training, working, knowledge, consulting services.

Note: ‘Technical assistance’ may involve the transfer of ‘technical data’.

‘Technical data’ may take forms such as blueprints, plans, diagrams, models, formulae, engineering designs and specifications, manuals and instructions written or recorded on other media or devices such as disk, tape, read-only memories.

‘Use’ is operation, installation (including on-site installation), maintenance (checking), repair,

overhaul and refurbishing.

# Justification

General part

Assessment of the current legal situation, an explanation of the necessity of the proposed legislation and a justification for its current principles

The list of selected items in the nuclear sector forms a part of Regulation No. 179/2002 Coll. laying down a list of selected items and dual-use items in the nuclear sector. Selected items are, according to Section 2(j)(2) of Act No. 18/1997 Coll. on the peaceful uses of nuclear energy and ionising radiation (the Atomic Act) and on amendments to other legislation, as amended – ‘materials, equipment and technology proposed and produced for use in the nuclear sector.’ Selected items include wrapped assemblies for irradiated nuclear fuel, hot chambers, nuclear reactors and specially designed or prepared equipment and components for their operation, deuterium, heavy water and graphite for use in a nuclear reactor, irradiated fuel element reprocessing plants, fuel element production plants, plants for the separation of uranium isotopes or the conversion of uranium or plutonium, and plants for the production or preparation of the concentration of heavy water.

Control of the selected items stems from Article III Paragraph 2 of the Nuclear Non-Proliferation Treaty (NPT), which was adopted by a resolution of the UN General Assembly on 12 June 1968; the Czech Republic is a contracting party to the Treaty. The export of the selected items is controlled by international inspection schemes such as the Zangger Committee and the Nuclear Suppliers Group (hereinafter ‘NSG’). The Czech Republic is an active participant in both of these schemes. The list of selected items specified by these Regulations is based on the NSG lists, which were published by the International Atomic Energy Agency (hereinafter ‘IAEA’) as INFCIRC/254/Rev.9/Part 1. These lists are updated regularly by the NSG.

Regulation No. 179/2002 Coll. was drawn up in accordance with the IAEA documents INFCIRC/254/Part 1 and Part 2, while the list of selected items was compiled according to Part 1 revision 5 and the list of dual-use items was prepared using Part 2 revision 4. The submitter proposes that the list of selected items and the list of dual-use items in the nuclear sector be split into two separate Regulations because INFCIRC/254/Part 1 and INFCIRC/254/Part 2 are separate IAEA documents that are updated separately by the NSG control scheme and then published independently of each other by the IAEA. The two lists are not updated at the same time: at present, revision 9 of  INFCIRC/254/Part 1 is in force, while INFCIRC/254/Part 2 has only reached revision 7. The two IAEA documents have completely different list structures, which is analogous to the two Regulations amending Regulation No. 179/2002 Coll., and they work differently with technical notes. This Regulation has been drawn up accordance with the latest revision of the IAEA document INFCIRC/254/Part 1, which is substantially different from revision 5, on the basis of which Regulation No. 179/2002 Coll. was prepared. A new section has been added, ‘8. General comments, technological control and definitions’, reflecting some generally-applicable principles when controlling the export of selected items in the nuclear sector. Some notes have been amended and clarified, and some new items have been added to the list, such as 5.2.3 ‘Special shut-off and control valves’, leading to the renumbering of Sections 5.2.3 and 5.2.4 such that national laws take into account all adjustments to the lists adopted by the NSG international inspection body. It is precisely the endeavour to facilitate timely amendments to national legislation based on the updating of only one of the INFCIRC/254 documents independently of the other that has lead the submitter to forward two separate Regulations for inter-departmental comment. For easier orientation in the matter of nuclear items, those items that, in addition to being selected items and dual-use items, are nuclear materials governed by separate regulations, will continue to be updated independently of each other, and moreover these amendments will more flexibly accept changes to the lists.

Assessment of the consistency of the draft legislation with the law it is to implement

The draft Regulation is in accordance with the Act No. 18/1997 Coll. on peaceful utilisation of nuclear energy and ionising radiation (the Atomic Act) and on amendments to other legislation, as amended, which it is proposed to implement, and it does not exceed the scope of legal authorisation.

Assessment of the consistency of the draft legislation with the constitutional order of the Czech Republic, international treaties, and the acts of the European Communities

The proposed legislation is consistent with the constitutional system of the Czech Republic and the international treaties to which the Czech Republic is bound.

The draft legislation is fully compatible with EU law, specifically Council Regulation (EC) No. 1334/2000 setting up a Community regime for the control of exports of dual-use items and technology, as amended, whose Annexes I and IV are updated in accordance with the changes to the international non-proliferation regimes and export control agreements, including the NSG.

Anticipated economic and financial implications of the draft legislation for the state budget, other public budgets, and the business environment in the Czech Republic.

The issuing of the Regulation will have no effect on the state budget of the Czech Republic and its entry into force will not increase the administrative burden on the public administration or its staffing levels.

The submitted draft Regulation will similarly place no increased financial burden on the other public budgets, and will not have a negative influence on the business environment in the Czech Republic.

Effects of the draft legislation on the environment

The proposed Regulation will not have a negative impact on the environment.

Social impact of the draft legislation

The proposed Regulation will not have a negative social impact.

With regard to the fact that the content of the submitted Regulation is the updating of an annex that is technical in character and that there will be no change to the substantive aspect of the regulations, the draft was exempted from an assessment of the effects of the regulations according to the General Principles approved by Government Resolution No. 877 of 13 August 2007.

### Specific part

Re § 1

Defines the subject of the Regulation and refers to the Annex, which specifically sets out the list of nuclear items.

Re § 2

This Regulation supersedes Regulation No. 179/2002 Coll., whose subject matter will continue to be governed by two separate decrees.

.

K § 3

This provision indicates the effectiveness of the Regulation. The actual date of entry into force will be determined during the course of the legislative proceedings.