KENYA STANDARD

Motor petrol (gasoline or spirit)–Specification

###### PUBLIC REVIEW DRAFT															NOVEMBER, 2005

KENYA BUREAU OF STANDARDS (KEBS)

## TECHNICAL COMMITTEE REPRESENTATION

## The Following organizations were represented in the Technical Committee.

## Ministry of Energy

## Mobil (k) Ltd

## African gas and oil co Ltd

## General Motors

## Kenya Petroleum Refineries Ltd.

## Kenya Pipeline

## National Oil Corporation of Kenya

### Total (K) Ltd

BP/Shell

## Kenya Bureau of Standards – Secretariat

## REVISION OF KENYA STANDARDS

## In order to keep abreast of progress in industry, Kenya Standards shall be regularly reviewed.  Suggestions for improvements to published standards, addressed to the Managing Director, Kenya Bureau of Standards are welcome.

## © Kenya Bureau of Standards, 2003

Copyright.  Users are reminded that by virtue of section 6 of the Copyright Act, Cap. 130 of the Laws of Kenya, copyright subsists in all Kenya Standards and except as provided under section 7 of this Act, no Kenya Standard produced by Kenya Bureau of Standards may be reproduced, stored in a retrieval system in any form or transmitted by any means without prior permission in writing from the Managing Director.

## KENYA BUREAU OF STANDARDS (KEBS)

## Head Office: P. O. Box 54974, Tel:(+254 02)502210-19,502543/45,Fax: (+254 02) 503293

## E-mail: info@kebs .org, web:http://www.kebs.org

## Coast Regional Office		Western Kenya Regional Office		Rift Valley Regional Office

## P.O. Box 99376, Mombasa	P.O. Box 2949, Kisumu			P.O. Box 8111, Eldoret

## Tel: (+254 011) 229563,230939/40	Tel: (+254 035) 23549, 22396		Tel: (+254 0321) 33151, 63377

## Fax: (+254 011) 229448		Fax: (+254 035) 21814			Fax: (+254 0321) 33150

## Foreword

This Kenya Standard has been prepared by the Fuels Technical Committee under the direction of the Petroleum Products Industry Standards Committee.

This is the second revision of the standard that was first passed in 1998. During Sub-Saharan Africa (SSA) Regional Conference in Dakar, Senegal held in June 2001, on the Launching of the Initiative for the Phase Out of Leaded Gasoline in SSA, a declaration was made, to which Kenya was signatory, that leaded gasoline be phased out of SSA latest December, 2005. A follow-up conference dubbed the East Africa Sub-Regional Conference in Nairobi, Kenya June 2002, SSA Government’s reiterated their commitment to complying with the Dakar Declaration and in line with these, adopted a minimum Research Octane Number, RON, of 91. Therefore, on this basis the oil industry and the Government have resolved that all gasoline produced in and imported into the country should be unleaded with effect from January 1, 2006.

In this standard, the maximum lead level has been specified at 0.015 g/l, while sulphur level has been specified as maximum 0.15 % m/m. In addition, use of oxygenates is strictly prohibited. The standard covers both Premium and Regular unleaded motor petrol.

During the revision of this standard, the following publications were referred and the assistance derived from them is acknowledged with thanks.

BS EN 228:1998. Specification for unleaded fuel.

SABS 1598:Specification for unleaded fuel.

AS 1876:Specification for leaded and unleaded gasoline.

Motor Petrol-Specification

Part 2: Unleaded Motor Petrol (Gasoline or Spirit)

- SCOPE

This Kenya standard specifies requirements and methods of test for unleaded motor petrol.

Note. The words motor petrol, motor gasoline and motor spirit can be used interchangeably.

- APPLICATION.

It applies to unleaded motor petrol for use in spark ignition engines, including those equipped with devices to reduce emitted pollutants. The standard applies to motor petrol as imported and marketed.

All requirements apply equally to regular and premium grade unleaded motor petrol unless indicated otherwise. The requirements regarding the regular grade apply only if a regular grade unleaded motor petrol is marketed. It does not apply to fuel used in jet engines.

- SAMPLING

Samples shall be taken as described in ISO 3170 or ISO 3171.

NOTE

It is essential that for sampling of unleaded petrol the containers used to take and store the samples before test be not contaminated with lead.

- Requirements and Test Methods

  - Dyes and Markers

The use of dyes and markers is allowed. The dye content shall be reported and shall not exceed 1.3mg/m3

  - Additives

In order to improve the performance quality the use of additives is allowed. Fuel additives without known harmful side effects are recommended in the appropriate amount, to help to avoid deterioration of driveability and emissions control durability. Other technical means with equivalent effect may be used

  - Phosphorous

In order to protect automotive catalyst systems, phosphorus-containing compounds shall not be blended into unleaded motor petrol.

  - Oxygenates content

The unleaded motor petrol shall not contain oxygenated compounds and a declaration to this effect shall be made.

  - Adulteration

The unleaded motor petrol shall not be adulterated with other petroleum products such as kerosene and diesel. This shall be determined through distillation (ASTM D86)

Distillation (table 1)

| % Evaporated | Temperature ºC |
| --- | --- |
| 10 | 71 |
| 50 | To be reported |
| 90 | To be reported |
| Final | 210 |

  - Generally Applicable Requirements and test methods

When tested in accordance with the test methods indicated in tables 2 and 3, premium grade unleaded motor petrol and regular grade unleaded motor petrol respectively shall be in accordance with the limits specified in tables 2 and 3

Table 2 Generally applicable requirements and test methods for premium grade unleaded motor petrol

| Property | Units | Requirements |  | Test method |
| --- | --- | --- | --- | --- |
|  |  | Min | Max |  |
| Research octane number RON |  | 91 | - | ISO 5164/ASTM D2699 |
| Motor octane number, MON |  | 83 | - | ISO 5163 |
| Lead content | g/l | - | 0.015 | EN 237/ASTM D3237 |
| Benzene content | %v/v | - | 5.0 | EN 238 |
| Density (at 150C) | kg/m3 | - | 780 | ISO 3675/ASTM D4052 |
| Sulfur content | %M/M | - | 0.15 | EN 24260/ISO 8754/ASTM D2622 |
| Oxidation stability | min | 360 | - | ISO 7536 |
| Existent gum content (solvent washed) | Mg/ 100ml | - | 5 | EN 5/ASTM D381 |
| Copper strip corrosion (3h at 500C) | rating | - | No. 1 strip | ISO 2160 |
| Appearance |  | Clear and bright |  | Visual inspection |
| Mercaptan sulphur | M/M | 0.002 | 0.002 | ASTM D1323 |
| Color | - | Red or orange |  | Visual inspection |
| Dye content | mg/m3 | 1.3 |  | - |
| Odour |  | Marketable |  | - |
| Oxygenates |  | Shall not be detected (declaration of absence shall be made) |  |  |
| Volatility |  | 93Kpa max |  |  |

Table 3,Generally applicable requirements and test methods for regular grade unleaded motor petrol

| Property | Units |  | Requirements |  | Test method |
| --- | --- | --- | --- | --- | --- |
|  |  |  | min | max |  |
| Research octane number RON |  |  | 87 | - | ISO 5164/ASTM D2699 |
| Motor octane number, MON |  |  | 77 | - | ISO 5163 |
| Lead content | g/l |  | - | 0.015 | EN 237/ASTM D3237 |
| Benzene content | %v/v |  | - | 5.0 | EN 238 |
| Density (at 150C) | kg/m3 |  |  | 780 | ISO 3675/ASTM D4052 |
| Sulphur content | % |  | - | 0.15 | EN 24260/ISO 8754/ASTM D2622 |
| Oxidation stability | min |  | 360 | - | ISO 7536 |
| Existent gum content (solvent washed) | m/g 100ml |  | - | 5 | EN 5/ASTM D381 |
| Copper strip corrosion (3h at 500C) | rating |  | No. 1 strip |  | ISO 2160/ASTM D130 |
| Appearance |  | - | Clear and bright |  | Visual inspection |
| Dye content |  | mg/m3 | 1.3 |  |  |
| Mercaptan sulphur |  | M/M | 0.002 |  | ASTM D1323 |
| Color | - |  | green |  | Visual inspection |
| Dye content | mg/m3 |  | 1.3 |  | - |
| Odour |  |  | Marketable |  | - |
| Oxygenates |  |  | Shall not be detected (declaration of absence shall be made) |  |  |
| Volatility |  |  | 93Kpa max |  |  |

4.7 Volatility

FVI=RVP+ 0.7E70=93 kpa maximum (RVP+0.IE70) = (13.5 psig maximum)

For both premium and regular motor petrol

Where

FVI-	Fuel vapour index

RVP-	Reid vapour pressure determined as per ASTM D323

E70-	fraction evaporated at 70ºC