COMMISSION DIRECTIVE 01/XX/EC of YYYYY amending Directive 94/2/EC implementing Council Directive 92/75/EEC with regard to energy labelling of household electric refrigerators, freezers and their combinations

(Text with EEA relevance)

THE COMMISSION OF THE EUROPEAN COMMUNITIES,

Having regard to the Treaty establishing the European Community,

Having regard to Council Directive 92/75/EEC of 22 September 1992 on the indication by labelling and standard product information of the consumption of energy and other resources of household appliances (1) and in particular Articles 9 and 12 thereof,

Having regard to Commission Directive 94/2/EC of 23 May 1995 implementing Council Directive 92/75/EEC with regard to energy labelling of household electric refrigerators, freezers and their combinations,

Whereas

(1) Directive 92/75/EEC requires the Commission to adopt implementing Directives in respect of various household appliances including refrigerators, freezers and their combinations.
(2) Electricity use by electric refrigerators, freezers and their combinations accounts for a significant part of total Community household energy demand. The further scope for a reduction of energy use by these appliances is substantial.
(3)The success of the existing labelling scheme (Directive 94/2) and the Minimum Efficiency Standard (Directive 96/57), have seen the efficiency index of new refrigerators and freezers rise by over 30% between 1996 and 2000.  This will mean that consumers will make lifetime electricity savings of the order of Euro 2.9 Billion on the appliances they bought in 2000.

(4) About a fifth appliances sold in 2000 were in the most efficient class (A), and in some markets the proportion was over one half. The market shares of A class appliances is rising rapidly.

(5) A continuing effect of labelling on energy efficiency will diminish, or disappear, unless further more efficient classes are defined.

(6) The energy plus scheme has already defined an informal energy threshold at an index of 42. The Cold II study showed that appliances with efficiency indices of 30 could be built now.

(7) Harmonised standards are technical specifications adopted by the European standardisation bodies, as listed in Annex I to Directive 98/34/EC of the European Parliament and of the Council of 22 June 1998 laying down a procedure for the provision of information in the field of technical standards and regulations and of rules on Information Society services(2), as amended by Directive 98/48/EC(3).
(8) Information concerning noise emissions should be given where required by Member States pursuant to Council Directive 86/594/EEC of 1 December 1986 on airborne noise emitted by household appliances(4).
(9) The measures provided for in this Directive are in accordance with the opinion of the committee set up under Article 10 of Directive 92/75/EEC,

HAS ADOPTED THIS DIRECTIVE:

Article 1

Directive 94/2/EC shall be amended as specified in the annex.

Article 2 
1. Member States shall adopt and publish the laws, regulations and administrative provisions to comply with this Directive by 31 December 2003. They shall immediately notify the Commission of these measures.

They shall allow the circulation of revised labels, and fiche, at least from 1 January 2004; and mail order catalogues at least from 1 July 2003.  They shall ensure that all labels, fiche, mail order catalogues and electronic communications comply with the revised models by 1 March 2004.

2. Member States shall communicate to the Commission any measures, which they take in the field covered by this Directive.

Article 3 
This Directive shall enter into force on the 20th day following its publication in the Official Journal of the European Communities.

Article 4 
This Directive is addressed to the Member States.

Done at Brussels, 17 December 1996.
For the Commission

Member of the Commission

(1) OJ No L 297, 13. 10. 1992, p. 16.
(2) OJ No L

Annex

Articles 1.2, 1.3 and 1.4 shall be replaced by the following: -

" 2. The information required by this Directive will be obtained by measurements made in accordance with harmonised standards adopted by the European Committee for Standardisation (CEN) under mandate from the Commission in accordance with Directive 98/34/EC, the reference numbers of which have been published in the Official Journal of the European Communities and for which Member States have published the reference numbers of the national standards transposing those harmonised standards

3. The provisions in the Annexes I, II and III to this Directive requiring the giving of information relating to noise shall apply only where that information is required by Member States under Article 3 of Directive 86/594/EEC. This information shall be measured in accordance with that Directive.

4. In this Directive expressions used have the same meaning as in Directive 92/75/EEC."

In Articles 2.1 the following indent shall be added at the end: -

"- Where the information relating to a particular model combination has been obtained by calculation on the basis of design, and/or extrapolation from other combinations, the documentation should include details of such calculations and/or extrapolations, and of tests undertaken to verify the accuracy of the calculations undertaken (Details of mathematical model for calculating performance and of measurements taken to verify this model)."

In Article 2.3 the following shall be added at the end: -

"; unless Member States make other provisions."

Article 2.5 shall be replaced by the following: -

"5. Where the appliances are offered for sale, hire or hire purchase by means of a printed or written communication, or by other means which imply that the potential customer cannot be expected to see the appliance displayed, such as a written offer, a mail order catalogue, advertisements on the Internet or on other electronic media, that communication shall include all the information specified in Annex III to this Directive."

In Article 2.6  "Annex V"   shall be replaced by "Annex IV".

In Annex I part 2 the final sentence "NB:….." shall be deleted.

In Annex I part 3 the following shall be inserted after the illustration:-

" The indicator letter for A+ and A++ appliances shall be in accordance with the following illustrations, and shall be placed in the same position as the A indicator for A class appliances.

A+							A++

"

and the text  "Complete printing …….to   …….B-1049 Brussels" shall be deleted.

In Annex II

Point 3 shall be replaced by: -

"3. Type of appliance, in accordance with standards referred to in Article 1 (2)."

In point 4, "Annex V" shall be replaced by "Annex IV" and "A (most efficient)" by "A++ (most efficient)" wherever they occur.

In point 9, "compartment" shall be replaced by "compartments".

And the final sentence "Note:….." shall be deleted.

In Annex III, the final sentence "Note: ….." shall be deleted.

Annex IV and Annex VI shall be deleted.

Annex V shall be renamed Annex IV and replaced by the following: -

"ANNEX IV

ENERGY EFFICIENCY CLASS

An appliance shall be classified as A+ or A++, where the energy efficiency index plus (Iα) is within the ranges specified in table 1.

Table 1

| Energy Efficiency index α (Iα) | 'energy efficiency class' |
| --- | --- |
| 30 > Iα | A++ |
| 42 > Iα ≥ 30 | A+ |
| Iα ≥ 42 | A - G (see below) |

If an appliance is not A+ or A++, it shall be classified depending on the value of the energy efficiency index (I) given in table 2:

Table 2

| Energy Efficiency index I | 'energy efficiency class' |
| --- | --- |
| 55 > I | A |
| 75 > I ≥ 55 | B |
| 90 > I ≥ 75 | C |
| 100 > I ≥ 90 | D |
| 110 > I ≥ 100 | E |
| 125 > I ≥ 110 | F |
| I ≥ 125 | G |

In tables 1 and 2

Iα  =     ACSCα      100 			I    =     ACSC      100

Where

AC     =   annual energy consumption of appliance (in accordance with annex I, note V.

SCα    	=	M  x  Compartments(Vc x (25−Tc)20  x FF x CC x BI)   +  N + CH

and

SC    		=	M  x  Compartments(Vc x (25−Tc)20  x FF )    +   N + CH

Where

Vc is the net volume (in litres) of the compartment

Tc is the design temperature  (in °C)  of the compartment

The values of  M and N are given in table 3 and  the values of FF, CC , BI and CH are given in table 4

Table 3

| Type of appliance | Temperature of coldest compartment | M | N |
| --- | --- | --- | --- |
| 1/2/3 Larder Fridge/ Refrigerator chiller/ Refrigerator no star/ Wine cooler | >- 6°C | 0.233 | 245 |
| 4 Refrigerator * |  - 6°c * | 0.643 | 191 |
| 5 Refrigerator ** |  - 12°c ** | 0.450 | 245 |
| 6/7 Refrigerator ***/ Fridge-freezer *(***) |  - 18°c ***/ *(***) | 0.777 | 303 |
| 8 Upright freezer |  - 18°c *(***) | 0.539 | 315 |
| 9 Chest freezer |  - 18°c *(***) | 0.472 | 286 |
| 10 Multi-door or other appliance |  | (4) | (4) |

(4) For these appliances,  the temperature and star rating of the compartment with the lowest temperature will determine the values of M and N.  Appliances with -18°C *(***) compartments shall be considered as Fridge-freezers *(***).

Table 4

| Correction Factor | Value | Condition |
| --- | --- | --- |
| FF (frost free) | 1.2 | For 'frost free' (ventilated) frozen food compartments |
|  | 1 | Otherwise |
| CC (climate class) | 1.2 | For 'Tropical' appliances |
|  | 1.1 | For 'sub-Tropical' appliances |
|  | 1 | Otherwise |
| BI (built in) | 1.2 | For built in appliances of under 58 cm in width. |
|  | 1 | Otherwise |
| CH (chill compartment) | 50 | For appliances with a chill compartment of at least 15 litres |
|  | 0 | Otherwise |

"

Efficiency index 96.1 in 1996, 72.9 in 2000 improvement of 31.8% .

31.8% of 352kWh/year (cold II table 6.3 p 145) x 10.64 (15 year life discounted @ 5% pa - cold II table 5.5 p 108) x 0.13 Euro/kWh (cold II table 5.5 p 108) x 18.730 million sales (cold II table 2.5 p 11 - 1999)

('standard annual energy consumption α of appliance' )

(standard annual energy consumption α of appliance)

in accordance with standards referred to in Article 1 (2)

An appliance is 'built in' only if it is designed exclusively for installation within a kitchen cavity with a need of furniture finishing, and tested as such.